Wiring Diagram: Compact Stair-Climbing UGV

TN-IMPACT 2026, Problem Statement TN126203. Rocker-bogie UGV with ESP32, six DC motors on three TB6612FNG drivers, and an FPV camera link.

Parts list

| Ref | Qty | Part | Notes |
| --- | --- | --- | --- |
| BT1 | 1 | 3S Li-ion pack with BMS | 12 V nominal |
| F1 | 1 | Inline fuse | Rate above motor stall current |
| SW1 | 1 | Latching emergency stop switch | In series with +12 V |
| U1 | 1 | LM2596 buck converter module | Set output to 5 V |
| C1 | 1 | 1000 µF, 25 V electrolytic | Across 12 V rail, near drivers |
| C2 | 1 | 220 µF, 10 V electrolytic | Across 5 V bus |
| R1 / R2 | 1 + 1 | 30 kΩ / 10 kΩ resistors | Battery divider to GPIO34 |
| R3 / R4 | 1 + 1 | 10 kΩ / 20 kΩ resistors | HC-SR04 ECHO divider to GPIO18 |
| MCU | 1 | ESP32 DevKit | MicroPython |
| DRV1–3 | 3 | TB6612FNG driver boards | Shared control lines |
| M1–M6 | 6 | DC gear motors | L1/R1, L2/R2, L3/R3 |
| IMU | 1 | MPU6050 breakout | I2C, 3.3 V |
| US | 1 | HC-SR04 ultrasonic sensor | 5 V, ECHO needs divider |
| RF | 1 | RF receiver module | UART to GPIO16, 5 V |
| FPV | 1 + 1 | FPV camera + 5.8 GHz VTX | 5 V, independent of ESP32 |

ESP32 pin assignments

| Signal | GPIO | Connects to |
| --- | --- | --- |
| PWMA / AIN1 / AIN2 | 25 / 26 / 27 | All 3 drivers, channel A (left motors) |
| PWMB / BIN1 / BIN2 | 13 / 32 / 33 | All 3 drivers, channel B (right motors) |
| STBY | 14 | All 3 drivers |
| SDA / SCL | 21 / 22 | MPU6050 |
| TRIG | 5 | HC-SR04 |
| ECHO | 18 | HC-SR04 via R3/R4 divider |
| UART2 RX | 16 | RF receiver data output |
| ADC | 34 | Battery divider (R1/R2) |

Build notes

- Join all grounds at one point. This keeps motor noise away from the ESP32.

- The HC-SR04 ECHO pin outputs 5 V, so it must go through the R3/R4 divider before GPIO18.

- Each TB6612FNG channel handles about 1.2 A continuous. Check your motor stall current and size F1 to match.

- Add a 100 nF capacitor across each motor's terminals to reduce noise.

- C1, C2 and the battery divider (R1/R2) are listed above but not drawn in the pictorial diagram.

- Part values and GPIO numbers are suggested starting points. Verify them against your actual modules.